Boxuan YU & 20513790

Boxuan YU 20513790

Title: Drawing robot software—Program planning document

Drawing robot software—Program planning document

Outline of the Problem to be Solved

Main purpose:

The main purpose is developing a software that enables a writing robot to draw text automatically. The program should read text and convent it into motion commands which can control the robot via the RS232 serial connection.

The program should focus on file reading, data convention.

Inputs:

A definition file containing X Y coordinate data and control the pen up or down states

The text file from user

Outputs:

A series of G code commands which are transmitted to the Arduino by the virtual serial port.

Key Problem Components

1. Font Data Processing

- Read and parse SingleStrokeFont.txt to obtain stroke-based character data

- Store all font definitions in memory for fast access

- Extract coordinates (X, Y) and pen states from each character definition

2. Text Input and Scaling

- Prompt user to input text height (4–10 mm range)

- Calculate scaling factor: scale = height / 18

- Read text content from the user’s text file for drawing

3. G-code Generation

- Convert scaled coordinates into G-code commands (G0, G1, S0, S1000)

- Apply automatic spacing: 5 mm line spacing and suitable character spacing

- Ensure text fits within 100 mm width and 50 mm height limits

- Lift pen and return to origin (0,0) after drawing all text

4. Robot Communication

- Send G-code commands one by one through the virtual RS232 port

- Wait for Arduino’s “ture” response before sending the next command

- Handle communication errors or timeouts safely

Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| Fontdata | struct | To store stroke data from SingleStrokeFont file. |
| Textinput | char | To store the text string to be drawn that is read from the text file. |
| Scale | float | To scale the coordinates in a font file by height. |
| Userheight | int | User input the height value(4mm-10mm) |
| Linespace | float | Define the fixed space between the rows |
| Offset | float | The horizontal offset of each letter relative to the previous letter to prevent overlap. |
| Ardureceived | Bool | Indicating whether received ture messages from Arduino before go on next command. |

Extend table as required

Function Declarations

int Readfontfile(char*input, char*output) -- Read the coordinate and pen status data in the font definition file.

Parameters:

Input: filename – path to the font definition file (SingleStrokeFont).

Output: internal fontData structure populated in memory.

Return value :returns 1 if the file is read successfully, 0 if failed.

Int Readusertext(char*input,int*input2,char*output)

Parameters:

Input: textfile from user

Input: user defines the height(4-10mm)

Output: internal fontData structure populated in memory.

Return value: returns 1 if the file is read successfully, 0 if failed.

float ScaleFactor(float height)-- Calculate the scaling ratio based on the height input by the user

Parameters:

Input: height – user text height (range 4–10mm).

Output: none.

Return value: computed scaling ratio height / 18

void GenerateGCode(char character, float scale)-- Convert the font coordinates to the corresponding G-Code instructions

Parameters:

Input: character – ASCII character to draw

Output: G-code lines appended to an internal transmit buffer

Return value: none.

int HelpGenerateGCode(float X, float Y, int penState, char* commandBuffer)---- Don’t need to be defined it just help to understand better

Parameters:

Input: X X coordinate

Input:Y Y coordinate

Input: pen state (0=up, 1=down)

commandBuffer -- output, buffer for G-code command

Return value: returns length of generated command

int SendtoArduino(char*input)—send the G-code to Arduino and waiting the feedback

Parameters:

Input: gcode – null-terminated G-code command string to transmit

Output: none directly .

Return value: 1 if the command is sent and acknowledged, 0 on communication error.

Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| Readfontfile | Verify correct reading of font data | filename = SingleStrokeFont | Function returns 1; fontData array correctly populated with coordinates and pen states for all characters. |
| Readusertext | Verify any simple text file | Textfile: Hello | Returns the correct content |
| ScaleFactor | Check the scale | If input height=18 | Returns=1 |
| GenerateGCode | Verify Gcode for every character | Character=H | Output contains correct Gcode |
| SendtoArduino | Test when no Arduino what received | Disconnect the Arduino | Function returns 0 |
| main | Test complete program flow | Input text file height | Program read, scale, send G-code ,at last the pen should return original |

Extend table as required. Note that ‘Function’ includes main()

Flowchart(s)

All flow charts have been included as separate pdf

AI Statement

For this coursework, I use AI only for improve the grammar, spelling, and fluency.